# Fundamentals of Computer Architecture and System Software

# 1. Introduction

Computer architecture is the discipline concerned with the design, organization, and behavior of the components
that make up a computing system. At its most fundamental level, a computer consists of a processor that executes
instructions, memory that stores data and programs, and input/output devices that connect the machine to the
outside world. The interaction between these components, governed by protocols and interfaces defined at design
time, determines the overall performance, power consumption, and capability of the system.

Modern computers are built upon decades of accumulated engineering knowledge. The stored-program concept,
formalized by John von Neumann in 1945, established the architectural template that still underlies most
computers today: instructions and data reside together in the same memory space, and the processor fetches,
decodes, and executes one instruction after another in sequence, occasionally branching to non-sequential
addresses. Although this model has been extended and optimized in countless ways — through pipelining,
superscalar execution, out-of-order processing, and speculative execution — its fundamental structure
remains recognizable.

Understanding computer architecture is essential for software engineers who want to write efficient code,
for system administrators who need to configure hardware, and for anyone who builds or maintains software
infrastructure. Even when working at a high level of abstraction, decisions about data structures, memory
access patterns, and concurrency are ultimately constrained by the properties of the underlying hardware.

# 2. The Central Processing Unit

The central processing unit (CPU) is the engine of the computer. It retrieves instructions from memory,
decodes them to determine what operation is required, and executes that operation using its internal
functional units. This fetch-decode-execute cycle runs continuously at a rate determined by the processor's
clock frequency, which in modern chips typically ranges from 3.0 GHz to 5.5 GHz.

A modern processor contains multiple execution cores, each capable of running an independent thread of
execution. A high-end desktop CPU may have 8, 12, or even 24 cores, while server processors can exceed
100 cores per chip. Each core contains an arithmetic logic unit (ALU) for integer operations, a
floating-point unit (FPU) for fractional and real-number arithmetic, a set of fast storage locations
called registers, and dedicated control logic.

To bridge the speed gap between the processor and main memory, CPUs contain several levels of cache.
The L1 cache, integrated directly into each core, is the fastest and smallest, typically 32 to 64 KB.
The L2 cache, slightly slower but larger at 256 KB to 1 MB per core, serves as a secondary buffer.
The L3 cache is shared among all cores on the chip and ranges from 8 MB in budget processors to 64 MB
or more in workstation-class chips. When the processor needs data, it first checks L1, then L2, then L3,
and finally issues a request to main memory if the data is not found in any cache level.

Two key techniques boost the throughput of the instruction pipeline. Branch prediction estimates, in
advance, which path a conditional branch will take, allowing the processor to fetch and begin executing
instructions before the condition is evaluated. Out-of-order execution allows the processor to execute
independent instructions in a different order from their appearance in the program, keeping the execution
units busy even when some instructions are waiting for data.

# 3. Memory Systems

Memory in a computer system is organized as a hierarchy, trading off speed, capacity, and cost at each
level. At the top of the hierarchy sit the processor registers: a few dozen 64-bit locations that operate
at the full clock speed of the CPU and can be read or written in a single cycle. Below the registers are
the cache levels described above, followed by main memory, solid-state or hard-disk storage, and finally
networked or cloud storage.

Main memory, implemented as dynamic random-access memory (DRAM), is the working space of running
programs. A modern workstation typically contains 16 to 128 GB of RAM organized in dual-channel or
quad-channel configurations to increase memory bandwidth. The current DDR5 standard supports transfer
rates between 4,800 MT/s and 8,400 MT/s per channel, providing a theoretical peak bandwidth in the range
of 50 to 100 GB/s for a dual-channel system.

Although DRAM is much slower than cache — a cache miss that requires accessing main memory typically
costs 50 to 100 CPU cycles — it is far faster than secondary storage. A high-end NVMe solid-state drive
achieves sequential read speeds of around 7,000 MB/s, but random access latency is measured in
microseconds rather than nanoseconds. Magnetic hard drives are slower still, with random-access latency
in the range of 5 to 10 milliseconds.

Virtual memory is an abstraction that allows each process to operate as though it has exclusive access
to a large, contiguous address space, regardless of how much physical memory is actually available. The
operating system maps virtual addresses to physical addresses through a data structure called a page table.
When a process accesses a virtual address that is not backed by physical memory, the OS raises a page
fault, loads the required data from disk, and resumes the process. This mechanism enables the system to
run programs whose combined memory requirements exceed the available physical RAM, at the cost of
significantly increased latency whenever disk access is required.

# 4. Input/Output and Storage

Input/output (I/O) devices connect the computer to the outside world, allowing it to receive data from
keyboards, mice, network interfaces, sensors, and cameras, and to send data to displays, printers,
speakers, and network links. I/O devices communicate with the processor through controllers, which
translate between the device-specific signaling and the bus protocols used inside the computer.

The Peripheral Component Interconnect Express (PCIe) bus is the dominant interconnect for high-speed
devices in modern PCs and servers. PCIe 5.0, the current generation, provides a bandwidth of 4 GB/s
per lane. A device using 16 lanes — typical for a graphics card — therefore has a peak bandwidth of
64 GB/s in each direction.

Storage devices retain data even when power is removed, in contrast to volatile main memory. NVMe
solid-state drives connect directly to the PCIe bus, bypassing the older SATA interface and achieving
much higher throughput and lower latency. NAND flash memory, the technology underlying SSDs, stores
data as charge levels in floating-gate transistors organized into cells. Modern 3D NAND stacks over
200 layers of cells vertically to achieve capacities of several terabytes per drive.

Direct memory access (DMA) allows I/O devices to transfer data directly to or from main memory without
involving the CPU for each transfer. The device signals an interrupt when the transfer is complete,
and the CPU then processes the received data. DMA is essential for achieving the full potential
bandwidth of modern storage and network devices, since even a fast CPU cannot sustain the data rates
required for a 100 Gbps network interface using programmed I/O.

# 5. Computer Networks

A computer network is a system of interconnected nodes that exchange data according to agreed-upon
protocols. The internet, the largest network in existence, is an interconnection of millions of smaller
networks operated by organizations, governments, and service providers. Its architecture follows the
TCP/IP model, which divides the communication problem into four layers: the link layer handles local
delivery over a single network segment, the internet layer routes packets across multiple networks using
IP addresses, the transport layer provides end-to-end reliability (TCP) or low-overhead delivery (UDP),
and the application layer contains protocols like HTTP, SMTP, and DNS that serve specific user needs.

Local area networks (LANs) connect devices within a building or campus. Wired Ethernet, standardized
as IEEE 802.3, is the dominant LAN technology, with current deployments offering 1 Gbps to 100 Gbps
per port. Wi-Fi, standardized as IEEE 802.11, provides wireless connectivity with current-generation
Wi-Fi 7 delivering aggregate throughput over 40 Gbps in ideal conditions, though real-world performance
is lower due to interference, distance, and the shared nature of the radio medium.

Security in computer networks involves multiple layers of defense. Transport Layer Security (TLS)
encrypts application-layer data between endpoints, protecting it from eavesdropping and tampering.
Firewalls filter traffic based on source address, destination address, port, and protocol. Intrusion
detection systems analyze network traffic for patterns that suggest malicious activity. Virtual private
networks (VPNs) create encrypted tunnels across untrusted networks, allowing remote users to access
internal resources securely.

Network performance is characterized by three key metrics: bandwidth, the maximum rate at which data
can be transferred; latency, the time required for a packet to travel from source to destination; and
packet loss, the fraction of packets that are dropped due to congestion or errors. For interactive
applications such as voice and video conferencing, latency below 150 ms is generally required for
acceptable quality. For bulk data transfer, bandwidth and loss rate are more important than latency.

# 6. Operating Systems

An operating system (OS) is the layer of software that manages the hardware resources of a computer
and provides a stable, convenient interface for application programs. Without an OS, each application
would need to contain its own code for interacting with every piece of hardware it might use — an
impossible burden that would also make it impossible for multiple programs to share the same machine.

The most important resource the OS manages is the processor. The scheduler decides which process or
thread runs on each core at each moment, balancing competing objectives: responsiveness (interactive
tasks should receive CPU time quickly), throughput (background work should complete efficiently), and
fairness (no process should starve). Modern schedulers are preemptive: they can interrupt a running
process and switch to another without the cooperation of the running process, using a hardware timer
to trigger the switch.

Memory management is the second major OS responsibility. The OS maintains the page tables that implement
virtual memory, allocates physical memory pages to processes, and handles page faults. It also provides
the heap allocator that application programmers use through functions like malloc and free. Efficient
memory management is critical for system performance; memory leaks and fragmentation can degrade
performance over time and cause programs to fail.

The file system provides an organized, persistent store for data. Files are named sequences of bytes
stored on a storage device. Directories organize files into a hierarchical namespace. The file system
maintains metadata about each file — its name, size, creation time, modification time, and access
permissions — and implements the operations that applications use: create, read, write, delete, rename,
and change permissions. Modern file systems include journaling, which records pending changes before
committing them, so that a crash during a write operation does not corrupt the file system structure.

# 7. Performance and Optimization

Measuring and improving computer performance is both an art and a science. The most important
principle is that optimization should be guided by measurement: tools called profilers record where
a program spends its time, which functions are called most often, and where memory is most heavily
allocated. Optimizing code that is not a bottleneck wastes effort and can introduce bugs.

Memory access patterns have a profound effect on performance. Modern processors are far faster than
main memory, and a program that repeatedly accesses data scattered through a large memory region will
spend most of its time waiting for cache misses. Reorganizing data structures to improve spatial
locality — keeping related data close together in memory — can reduce cache miss rates dramatically
and yield speedups of 5x or more without changing the algorithm at all.

Parallel execution on multi-core processors offers another avenue for speedup, but it introduces
complexity. Shared data must be protected from concurrent modification using synchronization primitives.
Locks introduce contention and can cause threads to block while waiting. Lock-free algorithms use
atomic hardware instructions to achieve thread safety without blocking, but are difficult to design
correctly. The degree to which a program can benefit from multiple cores is limited by the fraction
of its execution that must remain sequential, a constraint formalized by Amdahl's Law.

Power consumption is increasingly important as a performance metric. In mobile devices and large data
centers alike, energy efficiency directly affects battery life and operating costs. Dynamic voltage
and frequency scaling (DVFS) allows processors to reduce their clock speed and supply voltage during
periods of low load, reducing power consumption quadratically with clock speed. Modern chips combine
high-performance cores with efficiency cores optimized for low power, choosing dynamically which type
of core to use based on the current workload.

## Table 1. Processor Specifications

| Processor | Cores | Base Freq (GHz) | L3 Cache (MB) | TDP (W) | Architecture |
| --- | --- | --- | --- | --- | --- |
| Intel Core i9-14900K | 24 (8P+16E) | 3.2 | 36 | 125 | Raptor Lake |
| AMD Ryzen 9 7950X | 16 | 4.5 | 64 | 170 | Zen 4 |
| Intel Xeon w9-3595X | 60 | 2.5 | 75 | 350 | Sapphire Rapids |
| AMD EPYC 9654 | 96 | 2.4 | 384 | 360 | Genoa |
| Apple M3 Max | 16 (12P+4E) | 4.05 | 48 | 92 | ARM v8.6 |
| Qualcomm Snapdragon X | 12 | 3.8 | 42 | 23 | Oryon |

# Conclusion

Computer architecture, memory systems, networking, and operating systems together form the substrate on
which all software runs. Proficiency in these areas helps engineers make better decisions at every level
of the stack, from the choice of data structures to the design of distributed systems. The field continues
to evolve rapidly: chiplet-based designs decompose a processor into separately manufactured tiles;
processing-in-memory architectures move computation closer to data; neuromorphic and quantum processors
explore fundamentally different computational models. Despite this diversity, the principles covered in
this document — locality, hierarchy, abstraction, and concurrency — remain relevant across all of them.